Checking lambda control Checking lambda probe heating for lambda probes Note: The lambda probe heating circuit is monitored by the self-diagnosis system, interrogate fault memory => Page 01-21. - ‒ Connect the vehicle diagnostic, testing and information system VAS 5051 or fault reader V.A.G 1551 and select engine electronics control unit 1 with "Address word" 01 => Page 01-15. For this purpose, the engine must be running at idling speed.

| → Indicated on display: | Rapid data transfer HELP. Select function XX. |
| --- | --- |
| - ‒ Press keys 0 and 8 for the function "Read measured value block" and confirm entry with Q key. |  |

| → Indicated on display: | Read measured value block. Enter display group number XXX. |
| --- | --- |
| - ‒ Press keys 0, 4 and 1 for "display group number 41" and confirm entry with Q key. |  |

| → Indicated on display: | Read measured value block 41 ⇒. 1 2 3 4. |
| --- | --- |
| - ‒ Check resistance of lambda probe heating. Display zones. 1; 2; 3; 4. Display group 41: Lambda probe heating, bank 1 at idling speed. Display; xxx kOhm; Htg. u.c.c. ON; xxx kOhm; Htg. d.c.c. ON. Display; Bank 1, probe 1; Condition of lambda probe heating upstream of catalytic converter; Bank 1, probe 2; Condition of lambda probe heating downstream of catalytic converter. Range; Htg. u.c.c. ON Htg. u.c.c. OFF; Htg. d.c.c. ON Htg. d.c.c. OFF. Specified value; Smaller than 2 kOhm; Htg. u.c.c. ON; Smaller than 2 kOhm; Htg. d.c.c. ON. Note; It may take a few minutes until specified value is attained. - ‒ Press the Ckey. - ‒ Press keys 0, 4 and 2 for "display group number 42" and confirm entry with Q key. Display zones. 1; 2; 3; 4. Display group 42: Lambda probe heating, bank 2 at idling speed. Display; xxx kOhm; Htg. u.c.c. ON; xxx kOhm; Htg. d.c.c. ON. Display; Bank 1, probe 2; Condition of lambda probe heating upstream of catalytic converter; Bank 2, probe 2; Condition of lambda probe heating downstream of catalytic converter. Range; Htg. u.c.c. ON Htg. u.c.c. OFF; Htg. d.c.c. ON Htg. d.c.c. OFF. Specified value; Smaller than 2 kOhm; Htg. u.c.c. ON; Smaller than 2 kOhm; Htg. d.c.c. ON. Note; It may take a few minutes until specified value is attained. If specified value is not attained: - ‒ Check the voltage supply to the lambda probe heating => Page 24-117. - ‒ Check lambda probe signal wiring and actuation => Page 24-122. Check voltage supply for lambda probe heating Fitting locations of 4-pin connectors for lambda probes => Page 24-6. - ‒ Check fuse for lambda probe heating. => Current Flow Diagrams, Electrical Fault-finding and Fitting Locations binder - ‒ Disconnect 4-pin connector of relevant lambda probe. |  |

| - ‒ → Using leads from V.A.G 1594, connect hand-held multimeter (voltage measurement range) to sockets 1 (positive) and 2 (earth). - ‒ Operate starter briefly. Specified value: approx. battery voltage If no voltage is present: - ‒ → Using leads from V.A.G 1594, connect hand-held multimeter (voltage measurement range) to socket 1 (positive) and vehicle earth. - ‒ Operate starter briefly. Specified value: approx. battery voltage If no voltage is present: - ‒ Check the wiring from socket 1 on the relevant lambda probe connector on the wiring harness via the fuse to the fuel pump relay for open circuit or short. |  |
| --- | --- |

| => Current Flow Diagrams, Electrical Fault-finding and Fitting Locations binder If the power supply is OK: - ‒ → Using leads from V.A.G 1594, connect hand-held multimeter (voltage measurement range) to socket 2 (earth actuation from engine control unit) and battery positive. - ‒ Start the engine. Specified value: approx. battery voltage, fluctuating Note: At certain operating points, the engine control unit "cycles" the earth for the lambda probe heating. This means that at these points, the earth is continuously switched on and off. For this reason, the voltage reading on the tester may fluctuate. - ‒ Switch ignition off. |  |
| --- | --- |

| If no voltage is present: - ‒ Connect test box V.A.G 1598/31 to wiring harness for engine control unit; do not connect the engine control unit => Page 24-12. - ‒ → Check for open circuit in the following wiring connections: Lambda probe upstream of catalytic converter: Bank 1 -G39 Connector on wiring harness, socket; Test box V.A.G 1598/31 socket. 2; 5 (earth). Lambda probe upstream of catalytic converter: Bank 2 -G108 Connector on wiring harness, socket; Test box V.A.G 1598/31 socket. 2; 4 (earth). Lambda probe downstream of catalytic converter: Bank 1 -G130 Connector on wiring harness, socket; Test box V.A.G 1598/31 socket. 2; 63 (earth). Lambda probe downstream of catalytic converter: Bank 2 -G131 Connector on wiring harness, socket; Test box V.A.G 1598/31 socket. 2; 6 (earth). Wire resistance: max. 1.5 Ohm - ‒ Rectify open circuit, if necessary. => Current Flow Diagrams, Electrical Fault-finding and Fitting Locations binder If wiring connections are OK. - ‒ Replace appropriate lambda probe. |  |
| --- | --- |