| Audi A8 | Fitting Locations | No. 201 / 1 |
| --- | --- | --- |
|  |  | Edition 02.2006 |

| Position of relays and fuses. Fuses in fuse holder, left dash panel. Fuse colours. |  |  |
| --- | --- | --- |

| 40 A - | orange |
| --- | --- |

| 30 A - | green |
| --- | --- |

| 25 A - | white |
| --- | --- |

| 20 A - | yellow |
| --- | --- |

| 15 A - | blue |
| --- | --- |

| 10 A - | red |
| --- | --- |

| 7,5 A - | brown |
| --- | --- |

| 5 A - | beige |
| --- | --- |

| 3 A - | purple |
| --- | --- |

| 1 A - | black |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 2 |
| --- | --- | --- |

Fuses in fuse holder, left dash panel

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component |  | Terminal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - | SB1 - Fuse 1 on fuse holder | 5A | - | J530 - | Garage door operation control unit | 15 |
| 2 | - | SB2 - Fuse 2 on fuse holder | 10A 5A* | - | J446 - | Parking aid control unit | 30 |
| 3 | - | SB3 - Fuse 3 on fuse holder | 5A | - | J446 - | Parking aid control unit | 15 |
| 4 | - | SB4 - Fuse 4 on fuse holder | 10A | - | J431 - V48 - V49 - | Headlight range control, control unit left headlight range control motor right headlight range control motor | 15 |
| 5 | - | SB5 - Fuse 5 on fuse holder | 10A 5A* | - | J285 - J533 - | Control unit in dash panel insert Data bus diagnostic interface | 30 |
| 6 | - | SB6 - Fuse 6 on fuse holder | 10A | - | J527 - | Steering column electronics control unit | 30 |
| 7 | - | SB7 - Fuse 7 on fuse holder | 5A | - | T16 - | Diagnosic connetion | 30 |
| 8 | - | SB8 - Fuse 8 on fuse holder | 5A | - | T16 - G266 - | Diagnostic connector Oil level and oil temperature sender* | 15 |
| 9 | - | SB9 - Fuse 9 on fuse holder | 10A 5A* | - | J104 - J527 - | ABS control unit with ESP Steering column electronics control unit | 15 |
| 10 | - | SB10 - Fuse 10 on fuse holder | 5A | - | J533 - G266 - | Data bus diagnostic interfaces Oil level and oil temperature sender** | 15 |
| 11 | - | Vacant |  | - |  |  |  |
| 12 | - | SB12 - Fuse 12 on fuse holder | 5A | - | F - | Brake light switch | 30 |
| 13 | - | SB13 - Fuse 13 on fuse holder | 10A | - | R36 - J526 - J412 - R115 - | Telephone transmitter and receiver unit Telephone and telematics control unit Mobile telephone operating electronics control unit Chip card reader for telephone | 30 |

| * - from model year 2004. ** - up to model year 2003. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 3 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component |  | Terminal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | - | SB14 -Fuse 14 on fuse holder | 5A | - | J608 - | Special vehicle control unit# | 30 |
| 15 | - | SB15 -Fuse 15 on fuse holder | 5A | - | E408 - | Entry and start authorisation button | 30 |
| 16 | - | SB16 -Fuse 16 on fuse holder | 10A | - |  | Rear-Seat-Entertainment | 15 |
| 17 | - | SB17 - Fuse 17 on fuse holder | 10A 5A* | - | J428 - Z47 - | Adaptive cruise control unit Sensor heater for adaptive cruise control system** | 15 |
| 18 | - | SB18 -Fuse 18 on fuse holder | 5A | - | Z20 - Z21 - | Left washer jet heater element Right washer jet heater element | 75 |
| 19 | - | SB19 - Fuse 19 on fuse holder | 15A 10A# | - | J711 - J608 - | Washer fluid heater relay** Special vehicle control unit# | 75 30 |
| 20 | - | SB20 -Fuse 20 on fuse holder | 5A | - | J502 - | Tyre pressure monitor control unit | 30 |
| 21 | - | SB21 -Fuse 21 on fuse holder | 5A | - | J608 - | Special vehicle control unit# | 15 |
| 22 | - | SB22 -Fuse 22 on fuse holder | 5A | - | F47 - | Brake pedal switch | 87 |
| 23 | - | SB23 -Fuse 23 on fuse holder | 5A | - |  | Card system Italy (Telepass) | 30 |
| 24 | - | SB24 - Fuse 24 on fuse holder | 15A | - | J4 - H2 - H7 - | Dual tone horn relay Treble tone horn Bass tone horn | 30 |
| 25 | - | SB25 -Fuse 25 on fuse holder | 40A | - | J400 - | Wiper motor control unit | 30 |
| 26 | - | SB26-Fuse 26 on fuse holder | 30A 15A# | - | U18 - U19 - J608 - | 12 V socket 2** 12 V socket 3** Special vehicle control unit# | 30 |
| 27 | - | SB27 -Fuse 27 on fuse holder | 25A | - | J104 - | ABS control unit | 30 |
| 28 | - | Vacant |  |  |  |  |  |
| 29 | - | SB29 -Fuse 29 on fuse holder | 1A | - |  | Switch illumination | 58s |

| * - from model year 2004. ** - up to model year 2004. # - from model year 2006. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 4 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component |  | Terminal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | - | Vacant |  | - |  |  |  |
| 31 | - | SB31 - Fuse 31 on fuse holder | 30A | - | J519 - | Onboard supply control unit | 30 |
| 32 | - | Vacant |  | - |  |  |  |
| 33 | - | SB33 - Fuse 33 on fuse holder | 25A | - | Z42 - | Rear left footwell heater element | 75 |
| 34 | - | Vacant |  | - |  |  |  |
| 35 | - | Vacant |  | - |  |  |  |
| 36 | - | Vacant |  | - |  |  |  |
| 37 | - | SB37 - Fuse 37 on fuse holder | 15A | - | J698 - | Refrigerator box** | 15 |
| 38 | - | SB38 - Fuse 38 on fuse holder | 30A | - | J519 - | Onboard supply control unit | 30 |
| 39 | - | SB39 - Fuse 39 on fuse holder | 15A 7.5A* | - | J386 - | Driver door control unit | 30 |
| 40 | - | SB40 - Fuse 40 on fuse holder | 25A | - | J519 - V123 - V124 - | Onboard supply control unit Steering column vertical adjustment motor Steering column axial adjustment motor | 30 |
| 41 | - | SB41 - Fuse 41 on fuse holder | 15A 7.5A* | - | J388 - | Rear left door control unit | 30 |
| 42 | - | SB42 - Fuse 42 on fuse holder | 25A | - | J518 - | Entry and start authorisation control unit | 30 |
| 43 | - | SB43 - Fuse 43 on fuse holder | 10A | - |  | Left headlight with cornering lights** | 15 |
| 44 | - | SB44 - Fuse 44 on fuse holder | 10A | - |  | Right headlight with cornering lights** | 15 |

| * - from model year 2004. ** - from model year 2006. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 5 |
| --- | --- | --- |

| Position of relays and fuses. Fuses in fuse holder, right dash panel. Fuse colours. |  |  |
| --- | --- | --- |

| 40 A - | orange |
| --- | --- |

| 30 A - | green |
| --- | --- |

| 25 A - | white |
| --- | --- |

| 20 A - | yellow |
| --- | --- |

| 15 A - | blue |
| --- | --- |

| 10 A - | red |
| --- | --- |

| 7,5 A - | brown |
| --- | --- |

| 5 A - | beige |
| --- | --- |

| 3 A - | purple |
| --- | --- |

| 1 A - | black |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 6 |
| --- | --- | --- |

Fuses in fuse holder, right dash panel

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component |  | Terminal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - | SC1-Fuse 1 on fuse holder | 5A | - | J540 - | Electric park and handbrake control unit | 15 |
| 2 | - | SC2 - Fuse 2 on fuse holder | 10A | - | J255 - | Climatronic control unit | 30 |
| 3 | - | SC3 - Fuse 3 on fuse holder | 5A | - | F189 - F305 - N110 - J624 - | Tiptronic switch Gear selector position P switch Selector lever lock solenoid Engine control unit 2 (6.0 l petrol engine) | 15 |
| 4 | - | SC4 - Fuse 4 on fuse holder | 15A | - | J217 - | Automatic gearbox control unit (multitronic®) 01J, 01T | 87 |
| 5 | - | SC5 - Fuse 5 on fuse holder | 15A | - | J299 - N112 - G70 - N18 - N145 - N382 - J338 - J724 - V157 - | Secondary air pump relay Secondary air inlet valve Air mass meter Exhaust gas recirculation valve Right electrohydraulic engine mounting solenoid valve Torque rod valve Throttle valve module Turbocharger 1 control unit Intake manifold flap motor | 87 |
| 6 | - | SC6 - Fuse 6 on fuse holder | 15A | - | G39 - G108 - | Lambda probe lambda probe 2 | 87 |
| 7 | - | SC7 - Fuse 7 on fuse holder | 15A 10A | - | G130 - G131 - N276 - N290 - | Lambda probe after catalytic converter Lambda probe 2 after catalytic converter Fuel pressure regulating valve (TDI) Fuel metering valve | 87 |

| Audi A8 | Fitting Locations | No. 201 / 7 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component |  | Terminal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | - | SC8 - Fuse 8 on fuse holder | 10A | - | N205 - N208 - N382 - N321 - N213 - N144 - J544 - J725 - V51 - V275 - | Inlet camshaft control valve 1 Inlet camshaft control valve 2 Torque rod valve Exhaust flap 1 valve Exhaust gas recirculation valve 2 Left electrohydraulic engine mounting solenoid valve Throttle valve module 2 Turbocharger 2 control unit continued coolant circulation pump Intake manifold flap 2 motor | 87 |
| 9 | - | SC9 - Fuse 9 on fuse holder | 5A | - | G238 - E265 - E128 - E129 - G65 - J255 - E256 - | Air quality sensor Rear Climatronic operating and display unit Heated left driver seat switch Heated right driver seat switch High pressure sender Climatronic control unit TCS and ESP button | 15 |
| 10 | - | SC10 -Fuse 10 on fuse holder | 10A | - | J197 - | Adaptive suspension control unit | 15 |
| 11 | - | SC11 -Fuse 11 on fuse holder | 5A | - | G397 - | Rain and light detector sensor | 30 |
| 12 | - | SC12 -Fuse 12 on fuse holder | 5A | - | J523 - | Front information display and operating unit control unit | 30 |
| 13 | - | SC13 -Fuse 13 on fuse holder | 10A | - | J528 - | Roof electronics control unit | 30 |
| 14 | - | SC14 - Fuse 14 on fuse holder | 5A | - | R41 - R92 - R129 - | CD changer CD ROM drive Video recorder / DVD player | 30 |
| 15 | - | SC15 -Fuse 15 on fuse holder | 5A | - | J644 - | Energy management control unit | 30 |
| 16 | - | SC16 -Fuse 16 on fuse holder | 5A 7.5A | - | N79 - N79 - | Heater element for crankcase breather (V8 TDI) (up to model year 05) Heater element for crankcase breather (V8 TDI) (from model year 06) | 87 |

| Audi A8 | Fitting Locations | No. 201 / 8 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component |  | Terminal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 17 | - | SC17 - Fuse 17 on fuse holder | 5A* | - | V7 - V177 - | Vacant Radiator fan Radiator fan 2 | 87 |
| 18 | - | SC18 - Fuse 18 on fuse holder | 5A | - | L34 - J234 - J706 - | Map reading light bulb (up to 12/02) Vacant (from → 201/3 ) Airbag control unit** Seat occupied recognition control unit (only USA)** | 15 |
| 19 | - | SC19 - Fuse 19 on fuse holder | 5A | - | G70 - | Air mass meter (TDI) | 15 |
| 20 | - | SC20 - Fuse 20 on fuse holder | 5A | - | J672 - J673 - J526 - | Driver seat ventilation control unit Front passenger seat ventilation control unit Telephone / telematics control unit (USA) | 75 |
| 21 | - | SC21 - Fuse 21 on fuse holder | 5A 15A | - | J271 - J623 - J248 - | Motronic current supply relay Engine control unit (petrol engine) Diesel direct injection system control unit | 30 |
| 22 | - | SC22 - Fuse 22 on fuse holder | 10A | - | V166 - J445 - | Fuel cooling pump (TDI) Fuel cooling pump relay | 30 |
| 23 | - | SC23 - Fuse 23 on fuse holder | 5A | - | F321 - | Parking brake contact switch | 30 |
| 24 | - | SC24 - Fuse 24 on fuse holder | 20A 10A* | - | J520 - | Onboard supply control unit 2 | 30 |
| 25 | - | SC25 - Fuse 25 on fuse holder | 15A | - | J217 - | Automatic gearbox control unit 09E | 30 |
| 26 | - | SC26 - Fuse 26 on fuse holder | 15A 10A* | - | J255 - E265 - | SClimatronic control unit Rear Climatronic operating and display unit | 30 |

| * - from model year 2004. ** - from model year 2005. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 9 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component |  | Terminal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 27 | - | SC27 - Fuse 27 on fuse holder | 20A | - | J245 - | Sliding sunroof adjustment control unit | 30 |
| 28 | - | SC28 - Fuse 28 on fuse holder | 15A 5A* | - | J623 - J248 - J17 - J49 - J569 - | Engine control unit Diesel direct injection system control unit Fuel pump relay Electric fuel pump 2 relay Brake servo relay | 15 |
| 29 | - | SC29 - Fuse 29 on fuse holder | 20A 15A* | - | N30.. - | Injectors | 87 |
| 30 | - | SC30 - Fuse 30 on fuse holder | 30A | - | N... - J623 - | Ignition coils with output stage Engine control unit | 87 |
| 31 | - | SC31 - Fuse 31 on fuse holder | 20A | - | G23 - N115 - V144 - | Right fuel pump Active charcoal filter system solenoid valve 2 (USA) Fuel system diagnostic pump (USA) | 87 |
| 32 | - | SC32 - Fuse 32 on fuse holder | 15A 10A* 5A | - | J217 - J217 - | Automatic gearbox control unit (multitronic®) 01J, 01T Automatic gearbox control unit 09E, 09L | 15 |
| 33 | - | SC33 -Fuse 33 on fuse holder | 25A | - | Z43 - | Rear right footwell heater element | 75 |
| 34 | - | SC34 - Fuse 34 on fuse holder | 20A | - | E265 - | Seat heating/rear seat ventilation Rear Climatronic operating and display unit | 30 |
| 35 | - | SC35 - Fuse 35 on fuse holder | 20A | - | J255 - J672 - J673 - | Climatronic control unit Seat heating/front seat ventilation Driver seat ventilation control unit Front passenger seat ventilation control unit | 30 |

| * - from model year 2004. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 10 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component |  | Terminal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | - | SC36 -Fuse 36 on fuse holder | 20A | - | U1 - | Front cigarette lighter | 30 |
| 37 | - | SC37 -Fuse 37 on fuse holder | 20A 30A 25A# | - | U9 - U18 - U19 - | Rear cigarette lighter 12 V socket 2** 12 V socket 3** | 30 |
| 38 | - | SC38 - Fuse 38 on fuse holder | 20A* | - | V284 - | Vacant Radiator fan 3* | 30 |
| 39 | - | SC39 - Fuse 39 on fuse holder | 15A 7.5A* | - | J387 - | Front passenger side door control unit | 30 |
| 40 | - | SC40 - Fuse 40 on fuse holder | 15A | - | J569 - V192 - | Brake servo relay Vacuum pump for brakes | 30 |
| 41 | - | SC41 - Fuse 41 on fuse holder | 15A 7.5A* | - | J389 - | Rear right door control unit | 30 |
| 42 | - | SC42 - Fuse 42 on fuse holder | 20A | - | J364 - | Auxiliary heater control unit | 30 |
| 43 | - | SC43 - Fuse 43 on fuse holder | 20A 30A* | - | J39 - | Headlight washer system relay | 30 |
| 44 | - | SC44 - Fuse 44 on fuse holder | 15A 30A* | - | U10 - V2 - | Trailer socket Fresh air blower (air conditioning system)* | 15 |

| * - from model year 2004. ** - from model year 2005. # - from model year 2006. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 11 |
| --- | --- | --- |

Position relay and fuse holder in electronics box plenum chamber

| 1 - | Glow plug relay (V8 TDI, ASE) - J52 |
| --- | --- |

| - | Secondary air pump relay - J299 |
| --- | --- |

| 2 - | Glow plug relay 2 (V8 TDI, ASE) - J495 |
| --- | --- |

| - | Secondary air pump relay 2 (only VR 12) - J545 |
| --- | --- |

| - | from model year 04 |
| --- | --- |

| - | Motronic current supply relay - J271 |
| --- | --- |

| - | Terminal 30 voltage supply relay - J317 |
| --- | --- |

| 3 - | Terminal 30 voltage supply relay - J317 |
| --- | --- |

| - | Motronic current supply relay - J271 |
| --- | --- |

| - | from model year 04 |
| --- | --- |

| - | Glow plug relay 2 (V8 TDI, ASE) - J495 |
| --- | --- |

| - | Secondary air pump relay 2 (only VR 12, BHT) - J545 |
| --- | --- |

| - | Automatic glow period control unit - J179 (V8 TDI, BVN) |
| --- | --- |

| 4 - | Automatic glow period control unit - J179 (V6 TDI, ASB, BNG) |
| --- | --- |

| - | Glow period control unit 2 - J703 (V8 TDI, BVN) |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 12 |
| --- | --- | --- |

| A - | Vacant |
| --- | --- |

| B - | Engine glow plug strip fuse(V8 TDI, ASE) - S39, 60A (from model year 2004 80A) |
| --- | --- |

| - | Glow plugs fuse (V6 TDI, ASB, BNG/V8 TDI, ASE, BVN) - S125, 80A (from model year 2004) |
| --- | --- |

| - | Secondary air pump fuse - |
| --- | --- |

| - | S130, 50A |
| --- | --- |

| C - | Glow plugs fuse (V8 TDI, ASE) - S125, 60A (from model year 2004 80A) |
| --- | --- |

| - | Glow plugs fuse 2 (V8 TDI, ASE, BVN) - S189, 80A (from model year 2004) |
| --- | --- |

| - | Secondary air pump fuse 2 (only VR12, BHT) - S284, 50A |
| --- | --- |

| - | Automatic gearbox fuse, run-on - S286 |
| --- | --- |

| D - | Vacant |
| --- | --- |

| E - | Vacant |
| --- | --- |

| F - | Vacant |
| --- | --- |

| G - | Vacant |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 13 |
| --- | --- | --- |

Position relay and fuse holder behind left dash panel

| 1 - | Adaptive suspension compressor relay - J403 |
| --- | --- |

| 2 - | Dual tone horn relay - J4 |
| --- | --- |

| 3 - | Terminal 15 voltage supply relay - J329 |
| --- | --- |

| 4 - | Washer fluid heater relay - J711 (up to model year 04) |
| --- | --- |

| 5 - | Taxi alarm remote control, control unit - J601 (from model year 06) |
| --- | --- |

| 6 - | Terminal 75 voltage supply relay 1 - J680 |
| --- | --- |

| 7 - | Coupling steering column switch - T17f, black (up to model year 03) |
| --- | --- |

| - | Vacant (rom model year 04) |
| --- | --- |

| 8 - | Coupling dash panel vent illumination - T17g, grey |
| --- | --- |

| 9 - | Coupling loudspeaker in dash panel - T6a, black |
| --- | --- |

| - | Coupling loudspeaker in dash panel - T10t, black (loudspeaker with servomotor) |
| --- | --- |

| 10 - | Coupling electronic steering column lock - T8a, black |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 14 |
| --- | --- | --- |

| A - | ABS control unit fuse 1 - S123, 60A |
| --- | --- |

| B - | Levelling system fuse - S110 40A |
| --- | --- |

| C - | Window regulator thermal fuse - S43, 40A (from model year 04 30A) |
| --- | --- |

| D - | Seat adjustment thermal fuse 1 - S44, 30A |
| --- | --- |

| E - | Seat adjustment thermal fuse 2 - S45, 30A |
| --- | --- |

| F - | Vacant |
| --- | --- |

| J - | Coupling driver airbag - T5a, yellow (up to model year 03) |
| --- | --- |

| - | Vacant (rom model year 04) |
| --- | --- |

| K - | Vacant |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 15 |
| --- | --- | --- |

Position relay and fuse holder in front right footwell

| 1.1 - | Radiator fan 3 relay 3 (6.0 l petrol engine) - J752 |
| --- | --- |

| 1.1 - | Fuel cooling pump relay (TDI) - J445 |
| --- | --- |

| 1.1 - | Brake servo relay (petrol engine) - J569 |
| --- | --- |

| 1.2 - | Additional coolant pump relay - J496 |
| --- | --- |

| 1.2 - | Charge air cooling pump relay (V8 TDI, BVN) - J536 |
| --- | --- |

| - | Continued coolant circulation relay - J151 |
| --- | --- |

| 2 - | Starter motor relay - J53 |
| --- | --- |

| 3 - | Starter motor relay 2 - J695 |
| --- | --- |

| 4 - | Fuel pump relay - J17 |
| --- | --- |

| - | Engine component current supply relay (V10, BSM) - J757 |
| --- | --- |

| 5 - | Headlight washer system relay - J39 |
| --- | --- |

| A - | Front passenger side window regulator fuse - S170, 30A |
| --- | --- |

| B - | Front passenger seat adjustment thermal fuse 1 - S46, 30A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 16 |
| --- | --- | --- |

| C - | Hot air blower single fuse - S25, 30A (up to model year 03) |
| --- | --- |

| - | Vacant (rom model year 04) |
| --- | --- |

| D - | Front passenger seat adjustment thermal fuse 2 - S80, 30A |
| --- | --- |

| E - | Vacant |
| --- | --- |

| F - | Vacant |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 17 |
| --- | --- | --- |

Assignment main fuse carrier right A-pillar

| 1 - | Radiator fan single fuse - S42, 60A |
| --- | --- |

| 1A - | Screw point radiator fan |
| --- | --- |

| 2 - | Radiator fan fuse for 2nd speed - S104, 60A |
| --- | --- |

| 2A - | Screw point radiator fan fuse for 2nd speed |
| --- | --- |

| 3 - | Terminal 30 screw point (battery) |
| --- | --- |

| 4 - | Terminal 30 screw point (starter) |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 18 |
| --- | --- | --- |

Position relay and fuse holder right in luggage compartment

| 1 - | Heated rear window relay - J9 |
| --- | --- |

| 2 - | Vacant |
| --- | --- |

| - | Current supply relay - J16 (from model year 05) |
| --- | --- |

| 3 - | Fuel pump relay (Common Rail-Diesel) - J17 |
| --- | --- |

| - | Electric fuel pump 2 relay (petrol engine) - J49 |
| --- | --- |

| - | Cold start relay for fuel pump - J748 (from model year 06) |
| --- | --- |

| A - | Heated rear window single fuse - S41, 40A |
| --- | --- |

| B - | Rear seat adjustment thermal fuse 1 - S335, 30A (up to model year 05) |
| --- | --- |

| - | Fuel pump fuse - S81, 40A (from model year 06) |
| --- | --- |

| C - | Rear seat adjustment thermal fuse 2 - S336, 30A |
| --- | --- |

| D - | Vacant |
| --- | --- |

| - | Communications fuse - S254, 15A (from model year 05) |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 19 |
| --- | --- | --- |

Position relay and fuse holder right in luggage compartment

| 1 - | Terminal 30, fuse 1 - S131, 110A for heated windscreen - Z2 |
| --- | --- |

| 1A - | Terminal 30a screw point heated windscreen - Z2 |
| --- | --- |

| 2 - | Terminal 30, fuse 2 - S132, 60A for radio, navigation, TV tuner |
| --- | --- |

| 2A - | Terminal 30a screw point |
| --- | --- |

| 3 - | Terminal 30, fuse 3 - S133, 150A for onboard supply |
| --- | --- |

| 3A - | Terminal 30a screw point (onboard supply) |
| --- | --- |

| 4 - | Terminal 30 screw point |
| --- | --- |

| 5 - | Terminal 30 screw point |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 20 |
| --- | --- | --- |

| Position relay and fuse holder right in luggage compartment. Fuse holder (up to model year 03). Fuse designation: SF. |  |  |
| --- | --- | --- |

| 1 - | Convenience system central control unit (left light) - J393, 20A |
| --- | --- |

| 2 - | Convenience system central control unit (right light) - J393, 10A |
| --- | --- |

| 3 - | Convenience system central control unit (power latching system) - J393, 20A |
| --- | --- |

| 4 - | Electric park and handbrake control unit - J540, left parking brake motor - V282, 30A |
| --- | --- |

| 5 - | Electric park and handbrake control unit - J540, right parking brake motor - V283, 30A |
| --- | --- |

| 6 - | Trailer detector control unit (left light) - J345, 15A |
| --- | --- |

| 7 - | Trailer detector control unit (right light) - J345, 15A |
| --- | --- |

| 8 - | Trailer detector control unit - J345, 10A |
| --- | --- |

| 9 - | Fuel pump - G23, 20A |
| --- | --- |

| 10 - | Fuel system pressurisation pump - G6, 20A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 21 |
| --- | --- | --- |

| Position relay and fuse holder right in luggage compartment. Fuse holder (from model year 04). Fuse designation: SF. |  |  |
| --- | --- | --- |

| 1 - | Fuel pump - G23, 20A |
| --- | --- |

| 2 - | Fuel system pressurisation pump - G6, 20A |
| --- | --- |

| 3 - | Trailer socket - U10, Kl. 15, 15A (up to model year 05) |
| --- | --- |

| - | Vacant (rom model year 06) |
| --- | --- |

| 4 - | Vacant |
| --- | --- |

| 5 - | Convenience system central control unit (rear left light) - J393, 20A |
| --- | --- |

| 6 - | Convenience system central control unit (rear right light) - J393, 10A |
| --- | --- |

| 7 - | Convenience system central control unit (power latching system) - J393, 20A |
| --- | --- |

| 8 - | Electric park and handbrake control unit - J540, left parking brake motor - V282, 30A |
| --- | --- |

| 9 - | Electric park and handbrake control unit - J540, right parking brake motor - V283, 30A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 22 |
| --- | --- | --- |

| 10 - | Trailer detector control unit (left light) - J345, 15A |
| --- | --- |

| 11 - | Trailer detector control unit (right light) - J345, 15A |
| --- | --- |

| 12 - | Trailer detector control unit - J345, 15A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 23 |
| --- | --- | --- |

| Position fuse holder on left in luggage compartment (up to model year 03). Fuse designation: SD. |  |  |
| --- | --- | --- |

| 1 - | Digital sound package control unit - J525, 30A |
| --- | --- |

| 2 - | Navigation system with CD drive control unit - J401, 5A |
| --- | --- |

| 3 - | TV tuner - R78, 10A |
| --- | --- |

| 4 - | Vacant |
| --- | --- |

| 5 - | Radio - R, 5A |
| --- | --- |

| 6 - | Middle loudspeaker in rear shelf - R148, 30A |
| --- | --- |

| 7 - | 12 V socket - U5, 20A |
| --- | --- |

| 8 - | Remote control receiver for auxiliary heating - R64, 5A |
| --- | --- |

| 9 - | Vacant |
| --- | --- |

| 10 - | Vacant |
| --- | --- |

| A1 - | Terminal 30 screw point (battery) |
| --- | --- |

| B1 - | Terminal 30 screw point |
| --- | --- |

| Audi A8 | Fitting Locations | No. 201 / 24 |
| --- | --- | --- |

| Position fuse holder on left in luggage compartment (from model year 04). Fuse designation: SD. |  |  |
| --- | --- | --- |

| 1 - | Vacant |
| --- | --- |

| 2 - | Vacant |
| --- | --- |

| 3 - | Vacant |
| --- | --- |

| 4 - | Vacant |
| --- | --- |

| 5 - | Digital sound package control unit - J525, 30A |
| --- | --- |

| 6 - | Navigation system with CD drive control unit - J401, 5A |
| --- | --- |

| 7 - | TV tuner - R78, 10A |
| --- | --- |

| 8 - | Vacant |
| --- | --- |

| 9 - | Radio - R, 5A |
| --- | --- |

| - | Digital radio - R147, 5A |
| --- | --- |

| - | Satellite radio - R146, 5A (USA) |
| --- | --- |

| 10 - | Middle loudspeaker in rear shelf - R148, 15A |
| --- | --- |

| 11 - | 12 V socket - U5, 20A |
| --- | --- |

| 12 - | Remote control receiver for auxiliary heating - R64, 5A |
| --- | --- |

| A1 - | Terminal 30 screw point (battery) |
| --- | --- |

| B1 - | Terminal 30 screw point |
| --- | --- |