| Fuses. Overview of fuses. |  |  |
| --- | --- | --- |

| 1 - | Fuses (SB) on fuse holder, left dash panel |
| --- | --- |

| q | Location → page 802/3 |
| --- | --- |

| q | Position of fuses, left-hand drive → page 802/4 |
| --- | --- |

| q | Position of fuses, right-hand drive → page 802/8 |
| --- | --- |

| 2 - | Relay and fuse holder (S), in electronics box, in plenum chamber |
| --- | --- |

| q | Position of fuses, left-hand drive → page 802/24 |
| --- | --- |

| q | Position of fuses, right-hand drive → page 802/25 |
| --- | --- |

| 3 - | Relay and fuse holder (S), behind left dash panel |
| --- | --- |

| q | Position of fuses, left-hand drive → page 802/26 |
| --- | --- |

| q | Position of fuses, right-hand drive → page 802/27 |
| --- | --- |

| 4 - | Fuses (SC) on fuse holder, right dash panel |
| --- | --- |

| q | Location → page 802/13 |
| --- | --- |

| q | Position of fuses, left-hand drive → page 802/14 |
| --- | --- |

| q | Position of fuses, right-hand drive → page 802/20 |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 2 |
| --- | --- | --- |

| 5 - | Fuses (S) on main fuse carrier, right A-pillar |
| --- | --- |

| q | Position of fuses → page 802/30 |
| --- | --- |

| 6 - | Relay and fuse holder (S) in front right footwell |
| --- | --- |

| q | Position of fuses, left-hand drive → page 802/28 |
| --- | --- |

| q | Position of fuses, right-hand drive → page 802/29 |
| --- | --- |

| 7 - | Relay and fuse holder right in luggage compartment |
| --- | --- |

| q | Position of fuses → page 802/31 |
| --- | --- |

| 8 - | Relay and fuse holder right in luggage compartment, fuses |
| --- | --- |

| q | Position of fuses → page 802/32 |
| --- | --- |

| 9 - | Relay and fuse holder (SF) right in luggage compartment, fuses |
| --- | --- |

| q | Position of fuses up to model year 2003 → page 802/33 |
| --- | --- |

| q | Position of fuses from model year 2004 → page 802/34 |
| --- | --- |

| q | Position of fuses, right-hand drive → page 802/36 |
| --- | --- |

| 10 - | Fuses (SD) on fuse holder, left in luggage compartment |
| --- | --- |

| q | Position of fuses, up to model year 2003 → page 802/38 |
| --- | --- |

| q | Position of fuses, from model year 2004 → page 802/39 |
| --- | --- |

| q | Position of fuses, right-hand drive → page 802/41 |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 3 |
| --- | --- | --- |

Location of fuses (SB)

Fuse colours

40A orange

30A green

25A white

20A yellow

15A blue

10A red

7,5A brown

5A beige

3A purple

1A black

| Audi A8 | Fitting Locations | No. 802 / 4 |
| --- | --- | --- |

Fuses in fuse holder, left dash panel

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | - | SB1 - Fuse 1 on fuse holder B | 5 A | - | J530 - Garage door operation control unit | 15 |
| 2 | - | SB2 - Fuse 2 on fuse holder B | 10 A 5 A¹) | - | J446 - Parking aid control unit | 30 |
| 3 | - | SB3 - Fuse 3 on fuse holder B | 5 A | - | J446 - Parking aid control unit | 15 |
| 4 | - | SB4 - Fuse 4 on fuse holder B | 10 A | - | J431 - Headlight range control, control unit V48 - Left headlight range control motor V49 - Right headlight range control motor | 15 |
| 5 | - | SB5 - Fuse 5 on fuse holder B | 10 A 5 A¹) | - | J285 - Control unit in dash panel insert J533 - Data bus diagnostic interface²) | 30 |
| 6 | - | SB6 - Fuse 6 on fuse holder B | 10 A | - | J527 - Steering column electronics control unit | 30 |
| 7 | - | SB7 - Fuse 7 on fuse holder B | 5 A | - | T16 - 16-pin connector, diagnostic connector | 30 |
| 8 | - | SB8 - Fuse 8 on fuse holder B | 5 A | - | G266 - Oil level and oil temperature sender¹) T16 - 16-pin connector, diagnostic connector | 15 |
| 9 | - | SB9 - Fuse 9 on fuse holder B | 10 A 5 A¹) | - | J104 - ABS control unit J527 - Steering column electronics control unit | 15 |
| 10 | - | SB10 - Fuse 10 on fuse holder B | 5 A | - | G266 - Oil level and oil temperature sender²) J533 - Data bus diagnostic interface¹) | 15 |
| 11 | - | SB11 - Fuse 11 on fuse holder B | 10 A |  | J759 - Lane departure warning control unit³) | 15 |
| 12 | - | SB12 - Fuse 12 on fuse holder B | 5 A | - | F - Brake light switch | 30 |

| ¹) from model year 2004. ²) up to model year 2003. ³) from September 2007. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 5 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | - | SB13 - Fuse 13 on fuse holder B | 10 A | - | J412 -Mobile telephone operating electronics control unit J526 - Telephone and telematics control unit, telematics R36 - Telephone transmitter and receiver unit R115 - Chip card reader for telephone | 30 |
| 14 | - | SB14 - Fuse 14 on fuse holder B | 5 A | - | J608 - Special vehicle control unit³) | 30 |
| 15 | - | SB15 - Fuse 15 on fuse holder B | 5 A | - | E408 - Entry and start authorisation button | 30 |
| 16 | - | SB16 - Fuse 16 on fuse holder B | 10 A | - | - Rear-Seat-Entertainment | 15 |
| 17 | - | SB17 - Fuse 17 on fuse holder B | 10 A 5 A¹) | - | J428 - Adaptive cruise control unit Z47 - Sensor heater for adaptive cruise control system²) | 15 |
| 18 | - | SB18 - Fuse 18 on fuse holder B | 5 A | - | Z20 - Left washer jet heater element Z21 - Right washer jet heater element | 75 |
| 19 | - | SB19 - Fuse 19 on fuse holder B | 15 A 10 A³) | - | J608 - Special vehicle control unit³) J711 - Washer fluid heater relay²) | 30 75 |
| 20 | - | SB20 - Fuse 20 on fuse holder B | 5 A | - | J502 - Tyre pressure monitor control unit | 30 |
| 21 | - | SB21 - Fuse 21 on fuse holder B | 5 A | - | J608 - Special vehicle control unit³) | 15 |
| 22 | - | SB22 - Fuse 22 on fuse holder B | 5 A | - | F47 - Brake pedal switch | 87 |
| 23 | - | SB23 - Fuse 23 on fuse holder B | 5 A | - | - Card system Italy (Telepass) - Preparation for mobile telephone³) | 30 |
| 24 | - | SB24 - Fuse 24 on fuse holder B | 15 A | - | H2 - Treble tone horn H7 - Bass tone horn J4 - Dual tone horn relay | 30 |

| ¹) from model year 2004. ²) up to model year 2004. ³) from model year 2006. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 6 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 25 | - | SB25 - Fuse 25 on fuse holder B | 40 A | - | J400 - Wiper motor control unit | 30 |
| 26 | - | SB26 - Fuse 26 on fuse holder B | 15 A³) 30 A²) | - | J608 - Special vehicle control unit³) U18 - 12 V socket -2-²) U19 - 12 V socket 3²) | 30 |
| 27 | - | SB27 - Fuse 27 on fuse holder B | 25 A | - | J104 - ABS control unit | 30 |
| 28 | - | Vacant |  |  |  |  |
| 29 | - | SB29 - Fuse 29 on fuse holder B | 1 A | - | - Switch illumination | 58s |
| 30 | - | Vacant |  |  |  |  |
| 31 | - | SB31 - Fuse 31 on fuse holder B | 30 A | - | J519 - Onboard power supply control unit | 30 |
| 32 | - | Vacant |  |  |  |  |
| 33 | - | SB33 - Fuse 33 on fuse holder B | 25 A | - | Z42-Rear left footwell heater element | 75 |
| 34 | - | Vacant |  |  |  |  |
| 35 | - | Vacant |  |  |  |  |
| 36 | - | SB36 - Fuse 36 on fuse holder B | 5 A |  | J769 - Lane change assist control unit⁴) | 15 |
| 37 | - | SB37 - Fuse 37 on fuse holder B | 15 A | - | J698 - Refrigerator box³) | 15 |
| 38 | - | SB38 - Fuse 38 on fuse holder B | 30 A | - | J519 - Onboard power supply control unit | 30 |
| 39 | - | SB39 - Fuse 39 on fuse holder B | 15 A 7.5 A¹) | - | J386 - Driver door control unit | 30 |

| ¹) from model year 2004. ²) up to model year 2004. ³) from model year 2006. ⁴) from September 2007. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 7 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 40 | - | SB40 - Fuse 40 on fuse holder B | 25 A | - | J519 - Onboard power supply control unit V123 - Steering column vertical adjustment motor V124 - Steering column axial adjustment motor | 30 |
| 41 | - | SB41 - Fuse 41 on fuse holder B | 15 A 7.5 A¹) | - | J388 - Rear left door control unit | 30 |
| 42 | - | SB42 - Fuse 42 on fuse holder B | 25 A | - | J518 - Entry and start authorisation control unit | 30 |
| 43 | - | SB43 - Fuse 43 on fuse holder B | 10 A |  | - Left headlight with cornering lights²) | 15 |
| 44 | - | SB44 - Fuse 44 on fuse holder B | 10 A | - | - Right headlight with cornering lights²) | 15 |

| ¹) from model year 2004. ²) from model year 2006. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 8 |
| --- | --- | --- |

Position of fuses on fuse holder, left dash panel, right-hand drive

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | - | SB1 - Fuse 1 on fuse holder B | 5 A | - | J530 - Garage door operation control unit | 15 |
| 2 | - | SB2 - Fuse 2 on fuse holder B | 5 A | - | J446 - Parking aid control unit | 30 |
| 3 | - | SB3 - Fuse 3 on fuse holder B | 5 A | - | J446 - Parking aid control unit | 15 |
| 4 | - | SB4 - Fuse 4 on fuse holder B | 15 A | - | J217 - Automatic gearbox control unit 01J/01T | 87 |
| 5 | - | SB5 - Fuse 5 on fuse holder B | 15 A | - | G70 - Air mass meter J299 - Secondary air pump relay J338 - Throttle valve module J724 - Turbocharger 1 control unit N18 - Exhaust gas recirculation valve N112 - Secondary air inlet valve N145 - Right electrohydraulic engine mounting solenoid valve N382 - Torque rod valve V157 - Intake manifold flap motor | 87 |
| 6 | - | SB6 - Fuse 6 on fuse holder B | 15 A | - | G39 - Lambda probe G108 - Lambda probe 2 | 87 |
| 7 | - | SB7 - Fuse 7 on fuse holder B | 15 A 10 A | - | G130 - Lambda probe after catalytic converter G131 - Lambda probe 2 after catalytic converter N276 - Fuel pressure regulating valve N290 - Fuel metering valve | 87 |

| Audi A8 | Fitting Locations | No. 802 / 9 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 8 | - | SB8 - Fuse 8 on fuse holder B | 10 A | - | J544 - Throttle valve module 2 J725 - Turbocharger 2 control unit N144 - Left electrohydraulic engine mounting solenoid valve N205 - Inlet camshaft control valve 1 N208 - Inlet camshaft control valve 2 N213 - Exhaust gas recirculation valve 2 N321 - Exhaust flap 1 valve N382 - Torque rod valve V51 - Continued coolant circulation pump V275 - Intake manifold flap 2 motor | 87 |
| 9 | - | SB9 - Fuse 9 on fuse holder B | 5 A | - | J104 - ABS control unit J527 - Steering column electronics control unit | 15 |
| 10 | - | SB10 - Fuse 10 on fuse holder B | 10 A | - | J197 - Adaptive suspension control unit | 15 |
| 11 | - | SB11 - Fuse 11 on fuse holder B | 10 A | - | J759 - Lane departure warning control unit | 15 |
| 12 | - | SB12 - Fuse 12 on fuse holder B | 5 A | - | J523 - Front information display and operating unit control unit | 30 |
| 13 | - | SB13 - Fuse 13 on fuse holder B | 10 A | - | J412 -Mobile telephone operating electronics control unit J526 - Telephone and telematics control unit, telematics R36 - Telephone transmitter and receiver unit R115 - Chip card reader for telephone -Preparation for mobile telephone | 30 |
| 14 | - | SB14 - Fuse 14 on fuse holder B | 5 A | - | R41 - CD changer R92 - CD ROM drive R129 - Video recorder and DVD player | 30 |

| Audi A8 | Fitting Locations | No. 802 / 10 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | - | SB15 - Fuse 15 on fuse holder B | 5 A | - | E408 - Entry and start authorisation button | 30 |
| 16 | - | SB16 - Fuse 16 on fuse holder B | 7.5 A | - | N79 - Heater element for crankcase breather | 87 |
| 17 | - | SB17 - Fuse 17 on fuse holder B | 5 A | - | J428 - Adaptive cruise control unit Z47 - Sensor heater for adaptive cruise control system | 15 |
| 18 | - | SB18 - Fuse 18 on fuse holder B | 5 A | - | Z20 - Left washer jet heater element Z21 - Right washer jet heater element | 75 |
| 19 | - | SB19 - Fuse 19 on fuse holder B | 5 A | - | G70 - Air mass meter | 15 |
| 20 | - | SB20 - Fuse 20 on fuse holder B | 5 A | - | J502 - Tyre pressure monitor control unit | 30 |
| 21 | - | SB21 - Fuse 21 on fuse holder B | 15 A 5 A | - | J248 - Diesel direct injection system control unit J317 - Terminal 30 voltage supply relay J271 - Motronic current supply relay J623 - Engine control unit | 87 30 |
| 22 | - | SB22 - Fuse 22 on fuse holder B | 10 A | - | J445 - Fuel cooling pump relay V166 - Fuel cooling pump | 30 |
| 23 | - | SB23 - Fuse 23 on fuse holder B | 5 A | - | -Preparation for mobile telephone | 15 |
| 24 | - | SB24 - Fuse 24 on fuse holder B | 15 A | - | H2 - Treble tone horn H7 - Bass tone horn J4 - Dual tone horn relay | 30 |
| 25 | - | SB25 - Fuse 25 on fuse holder B | 15 A | - | J217 - Automatic gearbox control unit | 30 |
| 26 | - | Vacant |  |  |  |  |
| 27 | - | SB27 - Fuse 27 on fuse holder B | 25 A | - | J104 - ABS control unit | 30 |

| Audi A8 | Fitting Locations | No. 802 / 11 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 28 | - | SB28 - Fuse 28 on fuse holder B | 5 A | - | J17 - Fuel pump relay J49 - Electric fuel pump 2 relay J248 - Diesel direct injection system control unit J569 - Brake servo relay J623 - Engine control unit | 15 |
| 29 | - | SB29 - Fuse 29 on fuse holder B | 15 A | - | N30.. - Injector | 87 |
| 30 | - | SB30 - Fuse 30 on fuse holder B | 30 A | - | N...-Ignition coils with output stage J623 - Engine control unit | 87 |
| 31 | - | SB31 - Fuse 31 on fuse holder B | 20 A 40 A | - | G23 - Right fuel pump N115 - Activated charcoal filter solenoid valve 2 (USA) V144 - Fuel system diagnostic pump (USA) J538 - Fuel pump control unit | 87 |
| 32 | - | SB32 - Fuse 32 on fuse holder B | 10 A 5 A | - | J217 - Automatic gearbox control unit 01J, 01T J217 - Automatic gearbox control unit 09E, 09L | 15 |
| 33 | - | SB33 - Fuse 33 on fuse holder B | 25 A | - | Z42-Rear left footwell heater element | 75 |
| 34 | - | Vacant |  |  |  |  |
| 35 | - | Vacant |  |  |  |  |
| 36 | - | SB36 - Fuse 36 on fuse holder B | 5 A | - | J769 - Lane change assist control unit | 15 |
| 37 | - | SB37 - Fuse 37 on fuse holder B | 15 A | - | J698-Refrigerator box | 15 |
| 38 | - | SB38 - Fuse 38 on fuse holder B | 20 A | - | V284 - Radiator fan 3 | 30 |
| 39 | - | SB39 - Fuse 39 on fuse holder B | 7.5 A | - | J386 - Driver door control unit | 30 |

| Audi A8 | Fitting Locations | No. 802 / 12 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 40 | - | SB40 - Fuse 40 on fuse holder B | 15 A | - | J569 - Brake servo relay V192 - Vacuum pump for brakes | 30 |
| 41 | - | SB41 - Fuse 41 on fuse holder B | 7.5 A | - | J388 - Rear left door control unit | 30 |
| 42 | - | SB42 - Fuse 42 on fuse holder B | 25 A | - | J518 - Entry and start authorisation control unit | 30 |
| 43 | - | SB43 - Fuse 43 on fuse holder B | 10 A | - | - Left headlight with cornering lights | 15 |
| 44 | - | SB44 - Fuse 44 on fuse holder B | 10 A | - | - Right headlight with cornering lights | 15 |

| Audi A8 | Fitting Locations | No. 802 / 13 |
| --- | --- | --- |

Location of fuses (SC)

Fuse colours

40A orange

30A green

25A white

20A yellow

15A blue

10A red

7,5A brown

5A beige

3A purple

1A black

| Audi A8 | Fitting Locations | No. 802 / 14 |
| --- | --- | --- |

Fuses in fuse holder, right dash panel

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | - | SC1 - Fuse 1 on fuse holder C | 5 A | - | J540 - Electric park and handbrake control unit | 15 |
| 2 | - | SC2 - Fuse 2 on fuse holder C | 10 A | - | J255 - Climatronic control unit | 30 |
| 3 | - | SC3 - Fuse 3 on fuse holder C | 5 A | - | F189 - Tiptronic switch F305 - Gear selector position P switch N110 - Selector lever lock solenoid | 15 |
| 4 | - | SC4 - Fuse 4 on fuse holder C | 15 A | - | J217 - Automatic gearbox control unit 01J/01T | 87 |
| 5 | - | SC5-Fuse 5 on fuse holder C | 15 A | - | G70 - Air mass meter J299 - Secondary air pump relay J338 - Throttle valve module J724 - Turbocharger 1 control unit N18 - Exhaust gas recirculation valve N112 - Secondary air inlet valve N145 - Right electrohydraulic engine mounting solenoid valve N382 - Torque rod valve V157 - Intake manifold flap motor | 87 |
| 6 | - | SC6 - Fuse 6 on fuse holder C | 15 A | - | G39 - Lambda probe G108 - Lambda probe 2 | 87 |
| 7 | - | SC7 - Fuse 7 on fuse holder C | 15 A 10 A | - | G130 - Lambda probe after catalytic converter G131 - Lambda probe 2 after catalytic converter N276 - Fuel pressure regulating valve N290 - Fuel metering valve | 87 |

| Audi A8 | Fitting Locations | No. 802 / 15 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 8 | - | SC8 - Fuse 8 on fuse holder C | 10 A | - | N205 - Inlet camshaft control valve 1 N208 - Inlet camshaft control valve 2 N382 - Torque rod valve N321 - Exhaust flap 1 valve N213 - Exhaust gas recirculation valve 2 N144 - Left electrohydraulic engine mounting solenoid valve J544 - Throttle valve module 2 J725 - Turbocharger 2 control unit V51 - Continued coolant circulation pump V275 - Intake manifold flap 2 motor | 87 |
| 9 | - | SC9 - Fuse 9 on fuse holder C | 5 A | - | G238 - Air quality sensor E265 - Rear Climatronic operating and display unit E128 - Heated left driver seat switch E129 - Heated right driver seat switch G65 - High-pressure sender J255 - Climatronic control unit E256 - TCS and ESP button | 15 |
| 10 | - | SC10 - Fuse 10 on fuse holder C | 10 A | - | J197 - Adaptive suspension control unit | 15 |
| 11 | - | SC11 - Fuse 11 on fuse holder C | 5 A | - | G397 - Rain and light detector sensor | 30 |
| 12 | - | SC12 - Fuse 12 on fuse holder C | 5 A | - | J523 - Front information display and operating unit control unit | 30 |
| 13 | - | SC13 - Fuse 13 on fuse holder C | 10 A | - | J528-Roof electronics control unit | 30 |

| Audi A8 | Fitting Locations | No. 802 / 16 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 14 | - | SC14 - Fuse 14 on fuse holder C | 5 A | - | R41 - CD changer R92 - CD ROM drive R129 - Video recorder and DVD player | 30 |
| 15 | - | SC15 - Fuse 15 on fuse holder C | 5 A | - | J644 - Energy management control unit | 30 |
| 16 | - | SC16 - Fuse 16 on fuse holder C | 5 A 7.5 A³) | - | N79 - Heater element for crankcase breather | 87 |
| 17 | - | SC17 - Fuse 17 on fuse holder C | 5 A¹) | - | - Vacant V7 - Radiator fan V177 - Radiator fan 2 | 87 |
| 18 | - | SC18 - Fuse 18 on fuse holder C | 5 A | - | - vacant (from January 2003) J234 - Airbag control unit²) J706 - Seat occupied recognition control unit (only USA)²) L34 - Map reading light bulb (up to December 2002) | 15 |
| 19 | - | SC19 - Fuse 19 on fuse holder C | 5 A | - | G70 - Air mass meter | 15 |
| 20 | - | SC20 - Fuse 20 on fuse holder C | 5 A | - | J526 - Telephone and telematics control unit, telematics (USA) J672 - Driver seat ventilation control unit J673 - Front passenger seat ventilation control unit | 75 |
| 21 | - | SC21 - Fuse 21 on fuse holder C | 15 A 5 A | - | J248 - Diesel direct injection system control unit J271 - Motronic current supply relay J623 - Engine control unit | 30 |
| 22 | - | SC22 - Fuse 22 on fuse holder C | 10 A | - | J445 - Fuel cooling pump relay V166 - Fuel cooling pump | 30 |

| ¹) from model year 2004. ²) from model year 2005. ³) from model year 2006. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 17 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 23 | - | SC23 - Fuse 23 on fuse holder C | 5 A | - | F234 - Parking brake pressure switch | 30 |
| 24 | - | SC24 - Fuse 24 on fuse holder C | 20 A 10 A¹) | - | J520 - Onboard supply control unit 2 | 30 |
| 25 | - | SC25 - Fuse 25 on fuse holder C | 15 A | - | J217 - Automatic gearbox control unit | 30 |
| 26 | - | SC26 - Fuse 26 on fuse holder C | 15 A 10 A¹) | - | E265 - Rear Climatronic operating and display unit J255 - Climatronic control unit | 30 |
| 27 | - | SC27 - Fuse 27 on fuse holder C | 20 A | - | J245 - Sliding sunroof adjustment control unit | 30 |
| 28 | - | SC29 - Fuse 29 on fuse holder C | 15 A 5 A¹) | - | J17 - Fuel pump relay J49 - Electric fuel pump 2 relay J248 - Diesel direct injection system control unit J569 - Brake servo relay J623 - Engine control unit | 15 |
| 29 | - | SC29 - Fuse 29 on fuse holder C | 20 A 15 A¹) | - | N30.. - Injector | 87 |
| 30 | - | SC30 - Fuse 30 on fuse holder C | 30 A | - | N...-Ignition coils with output stage J623 - Engine control unit | 87 |
| 31 | - | SC31 - Fuse 31 on fuse holder C | 20 A 40 A | - | G23 - Right fuel pump N115 - Activated charcoal filter solenoid valve 2 (USA) V144 - Fuel system diagnostic pump (USA) J538 - Fuel pump control unit | 87 |
| 32 | - | SC32 - Fuse 32 on fuse holder C | 15 A 10 A¹) 5 A²) | - | J217 - Automatic gearbox control unit 01J, 01T J217 - Automatic gearbox control unit 09E, 09L | 15 |

| ¹) from model year 2004. ²) from model year 2006. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 18 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 33 | - | SC33-Fuse 33 on fuse holder C | 25 A | - | Z43-Rear right footwell heater element | 75 |
| 34 | - | SC34 - Fuse 34 on fuse holder C | 20 A | - | - Seat heating/rear seat ventilation E265 - Rear Climatronic operating and display unit | 30 |
| 35 | - | SC35 - Fuse 35 on fuse holder C | 20 A | - | J255 - Climatronic control unit J672 - Driver seat ventilation control unit J673 - Front passenger seat ventilation control unit | 30 |
| 36 | - | SC36 - Fuse 36 on fuse holder C | 20 A | - | U1 - Cigarette lighter | 30 |
| 37 | - | SC37 - Fuse 37 on fuse holder C | 20 A 30 A 25 A³) | - | U9 - Rear cigarette lighter U18 - 12 V socket -2-²) U19 - 12 V socket 3²) | 30 |
| 38 | - | SC38 - Fuse 38 on fuse holder C | 20 A¹) | - | - Vacant V284 - Radiator fan 3¹) | 30 |
| 39 | - | SC39 - Fuse 39 on fuse holder C | 15 A 7.5 A¹) | - | J387 - Front passenger side door control unit | 30 |
| 40 | - | SC40 - Fuse 40 on fuse holder C | 15 A | - | J569 - Brake servo relay V192 - Vacuum pump for brakes | 30 |
| 41 | - | SC41 - Fuse 41 on fuse holder C | 15 A 7.5 A¹) | - | J389 - Rear right door control unit | 30 |

| ¹) from model year 2004. ²) from model year 2005. ³) from model year 2006. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 19 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 42 | - | SC42 - Fuse 42 on fuse holder C | 20 A | - | J364 - Auxiliary heater control unit | 30 |
| 43 | - | SC43 - Fuse 43 on fuse holder C | 20 A 30 A¹) |  | J39 - Headlight washer system relay | 30 |
| 44 | - | SC44 - Fuse 44 on fuse holder C | 15 A 30 A¹) | - | U10 -Trailer socket V2 - Fresh air blower (air conditioning)¹) | 15 |

| ¹) from model year 2004. |  |  |
| --- | --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 20 |
| --- | --- | --- |

Position of fuses on fuse holder, right dash panel, right-hand drive

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | - | SC1 - Fuse 1 on fuse holder C | 5 A | - | J540 - Electric park and handbrake control unit | 15 |
| 2 | - | SC2 - Fuse 2 on fuse holder C | 10 A | - | J255 - Climatronic control unit | 30 |
| 3 | - | SC3 - Fuse 3 on fuse holder C | 5 A | - | F189 - Tiptronic switch F305 - Gear selector position P switch N110 - Selector lever lock solenoid | 15 |
| 4 | - | SC4 - Fuse 4 on fuse holder C | 10 A | - | J431 - Headlight range control, control unit V48 - Left headlight range control motor V49 - Right headlight range control motor | 15 |
| 5 | - | SC5-Fuse 5 on fuse holder C | 5 A | - | J285 - Control unit in dash panel insert J533 - Data bus diagnostic interface | 30 |
| 6 | - | SC6 - Fuse 6 on fuse holder C | 10 A | - | J527 - Steering column electronics control unit | 30 |
| 7 | - | SC7 - Fuse 7 on fuse holder C | 5 A | - | T16 - 16-pin connector, diagnostic connector | 30 |
| 8 | - | SC8 - Fuse 8 on fuse holder C | 5 A | - | G226 - Oil level and oil temperature sender T16 - 16-pin connector, diagnostic connector | 15 |
| 9 | - | SC9 - Fuse 9 on fuse holder C | 5 A | - | E128 - Heated rear left seat switch with regulator E129 - Heated rear right seat switch with regulator E256 - TCS and ESP button E265 - Rear Climatronic operating and display unit G65 - High-pressure sender G238 - Air quality sensor J255 - Climatronic control unit | 15 |

| Audi A8 | Fitting Locations | No. 802 / 21 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | - | SC10 - Fuse 10 on fuse holder C | 5 A | - | J533 - Data bus diagnostic interface | 15 |
| 11 | - | SC11 - Fuse 11 on fuse holder C | 5 A | - | G397 - Rain and light detector sensor | 30 |
| 12 | - | SC12 - Fuse 12 on fuse holder C | 5 A | - | F - Brake light switch | 30 |
| 13 | - | SC13 - Fuse 13 on fuse holder C | 10 A | - | J528-Roof electronics control unit | 30 |
| 14 | - | Vacant |  |  |  |  |
| 15 | - | SC15 - Fuse 15 on fuse holder C | 5 A | - | J644 - Energy management control unit | 30 |
| 16 | - | SC16 - Fuse 16 on fuse holder C | 10 A | - | - Rear-Seat-Entertainment | 15 |
| 17 | - | SC17 - Fuse 17 on fuse holder C | 5 A | - | V7 - Radiator fan V177 - Radiator fan 2 | 87 |
| 18 | - | SC18 - Fuse 18 on fuse holder C | 5 A | - | J234 - Airbag control unit J706 - Seat occupied recognition control unit (only USA) | 15 |
| 19 | - | Vacant |  |  |  |  |
| 20 | - | SC20 - Fuse 20 on fuse holder C | 5 A | - | J672 - Driver seat ventilation control unit J673 - Front passenger seat ventilation control unit | 75 |
| 21 | - | Vacant |  |  |  |  |
| 22 | - | SC22 - Fuse 22 on fuse holder C | 5 A | - | F47 - Brake pedal switch | 87 |
| 23 | - | SC23 - Fuse 23 on fuse holder C | 5 A | - | F234 - Parking brake pressure switch | 30 |
| 24 | - | SC24 - Fuse 24 on fuse holder C | 10 A | - | J520 - Onboard supply control unit 2 | 30 |
| 25 | - | SC25 - Fuse 25 on fuse holder C | 40 A | - | J400 - Wiper motor control unit | 30 |

| Audi A8 | Fitting Locations | No. 802 / 22 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 26 | - | SC26 - Fuse 26 on fuse holder C | 10 A | - | E265 - Rear Climatronic operating and display unit J255 - Climatronic control unit | 30 |
| 27 | - | SC27 - Fuse 27 on fuse holder C | 20 A | - | J245 - Sliding sunroof adjustment control unit | 30 |
| 28 | - | Vacant |  |  |  |  |
| 29 | - | SC29 - Fuse 29 on fuse holder C | 1 A | - | - Switch illumination | 58s |
| 30 | - | Vacant |  |  |  |  |
| 31 | - | SC31 - Fuse 31 on fuse holder C | 30 A | - | J519 - Onboard power supply control unit | 30 |
| 32 | - | Vacant |  |  |  |  |
| 33 | - | SC33-Fuse 33 on fuse holder C | 25 A | - | Z43-Rear right footwell heater element | 75 |
| 34 | - | SC34 - Fuse 34 on fuse holder C | 20 A | - | - Seat heating/rear seat ventilation E265 - Rear Climatronic operating and display unit | 30 |
| 35 | - | SC35 - Fuse 35 on fuse holder C | 20 A | - | J255 - Climatronic control unit J672 - Driver seat ventilation control unit J673 - Front passenger seat ventilation control unit | 30 |
| 36 | - | SC36 - Fuse 36 on fuse holder C | 20 A | - | U1 - Cigarette lighter | 30 |
| 37 | - | SC37 - Fuse 37 on fuse holder C | 20 A 25 A | - | U9 - Rear cigarette lighter U18 - 12 V socket -2- U19 - 12 V socket 3 | 30 |
| 38 | - | SC38 - Fuse 38 on fuse holder C | 30 A | - | J519 - Onboard power supply control unit | 30 |

| Audi A8 | Fitting Locations | No. 802 / 23 |
| --- | --- | --- |

| No. |  | Current Flow Diagram designation | Nominal value |  | Function/component | Terminal |
| --- | --- | --- | --- | --- | --- | --- |
| 39 | - | SC39 - Fuse 39 on fuse holder C | 7.5 A | - | J387 - Front passenger side door control unit | 30 |
| 40 | - | SC40 - Fuse 40 on fuse holder C | 25 A | - | J519 - Onboard power supply control unit V123 - Steering column vertical adjustment motor V124 - Steering column axial adjustment motor | 30 |
| 41 | - | SC41 - Fuse 41 on fuse holder C | 7.5 A | - | J389 - Rear right door control unit | 30 |
| 42 | - | SC42 - Fuse 42 on fuse holder C | 20 A | - | J364 - Engine residual heat accumulator control unit | 30 |
| 43 | - | SC43 - Fuse 43 on fuse holder C | 30 A |  | J39 - Headlight washer system relay | 30 |
| 44 | - | SC44 - Fuse 44 on fuse holder C | 30 A | - | V2 - Fresh air blower (air conditioning) | 30 |

| Audi A8 | Fitting Locations | No. 802 / 24 |
| --- | --- | --- |

Fuse locations on fuse holder, in electronics box

| A - | Vacant |
| --- | --- |

| B - | Glow plug strip fuse -S39-, 60A |
| --- | --- |

| q | Engine code ASE only |
| --- | --- |

| q | from model year 2004 |
| --- | --- |

| B - | Glow plug fuse -S125-, 80A |
| --- | --- |

| q | Engine codes ASB, BNG, ASE, BVN only |
| --- | --- |

| q | from model year 2004 |
| --- | --- |

| B - | Secondary air pump fuse -S130-, 50A |
| --- | --- |

| C - | Glow plug fuse -S125-, 60A |
| --- | --- |

| q | Engine code ASE only |
| --- | --- |

| q | from model year 2004, 80A |
| --- | --- |

| C - | Glow plug fuse 2 -S189-, 80A |
| --- | --- |

| q | Engine codes ASE, BVN only |
| --- | --- |

| q | from model year 2004 |
| --- | --- |

| C - | Secondary air pump fuse 2 -S284-, 50A |
| --- | --- |

| q | Engine code BHT only |
| --- | --- |

| D - | Vacant |
| --- | --- |

| F - | Vacant |
| --- | --- |

| G - | Vacant |
| --- | --- |

| H - | Vacant |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 25 |
| --- | --- | --- |

Fuse locations on fuse holder, in electronics box, right-hand drive

| A - | Vacant |
| --- | --- |

| B - | Vacant |
| --- | --- |

| C - | Vacant |
| --- | --- |

| D - | Vacant |
| --- | --- |

| E - | Engine glow plug strip fuse -S39-, 80 A |
| --- | --- |

| q | Engine code ASE only |
| --- | --- |

| - | Glow plug fuse -S125-, 80 A |
| --- | --- |

| q | Engine codes ASE, BVN, ASB, BNG only |
| --- | --- |

| - | Secondary air pump fuse -S130-, 50 A |
| --- | --- |

| F - | Vacant |
| --- | --- |

| G - | Glow plug fuse 2 -S189-, 80 A |
| --- | --- |

| q | Engine codes ASE, BVN only |
| --- | --- |

| - | Secondary air pump fuse 2 -S284-, 50 A |
| --- | --- |

| q | Engine code BHT only |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 26 |
| --- | --- | --- |

Position of fuses on fuse holder, on left behind dash panel

| A - | ABS control unit fuse 1 -S123-, 60A |
| --- | --- |

| B - | Levelling system fuse -S110- 40A |
| --- | --- |

| C - | Window regulator thermal fuse -S43- 40A |
| --- | --- |

| q | from model year 2004, 30A |
| --- | --- |

| D - | Seat adjustment thermal fuse 1 -S44-, 30A |
| --- | --- |

| E - | Seat adjustment thermal fuse 2 -S45- 30A |
| --- | --- |

| F - | Vacant |
| --- | --- |

| G - | Vacant |
| --- | --- |

| H - | Vacant |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 27 |
| --- | --- | --- |

Position of fuses on fuse holder, on right behind dash panel, right-hand drive

| A - | Driver seat adjustment thermal fuse 1 -S44-, 30 A |
| --- | --- |

| B - | Window regulator thermal fuse -S43-, 30 A |
| --- | --- |

| C - | Vacant |
| --- | --- |

| F - | Vacant |
| --- | --- |

| G - | Driver seat adjustment thermal fuse 2 -S45-, 30 A |
| --- | --- |

| H - | Vacant |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 28 |
| --- | --- | --- |

Fuses on fuse holder, in front right footwell

| A - | Front passenger side window regulator fuse -S170-, 30 A |
| --- | --- |

| q | up to July 2006 |
| --- | --- |

| B - | Front passenger seat adjustment thermal fuse 1 -S46- 30 A |
| --- | --- |

| C - | Hot air blower single fuse -S25-, 30 A |
| --- | --- |

| q | up to model year 2003 |
| --- | --- |

| C - | Vacant |
| --- | --- |

| q | from model year 2004 |
| --- | --- |

| C - | Front passenger side window regulator fuse -S170-, 30 A |
| --- | --- |

| q | from August 2006 |
| --- | --- |

| D - | Front passenger seat adjustment thermal fuse 2 -S80-, 30 A |
| --- | --- |

| E - | Vacant |
| --- | --- |

| F - | Vacant |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 29 |
| --- | --- | --- |

Fuses on fuse holder, in front right footwell, right-hand drive

| A - | Window regulator thermal fuse -S43-, 30 A |
| --- | --- |

| q | up to October 2006 |
| --- | --- |

| B - | Driver seat adjustment thermal fuse 1 -S44-, 30 A |
| --- | --- |

| C - | Vacant |
| --- | --- |

| q | up to October 2006 |
| --- | --- |

| - | Window regulator thermal fuse -S43-, 30 A |
| --- | --- |

| q | from November 2006 |
| --- | --- |

| D - | ABS control unit fuse 1 -S123-, 60 A |
| --- | --- |

| E - | Driver seat adjustment thermal fuse 2 -S45-, 30 A |
| --- | --- |

| F - | Levelling system fuse -S110-, 40 A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 30 |
| --- | --- | --- |

Position of fuses on main fuse carrier, right A-pillar

| 1 - | Radiator fan single fuse -S42-, 60A |
| --- | --- |

| 1A - | Screw point radiator fan |
| --- | --- |

| 2 - | Radiator fan fuse for 2nd speed -S104-, 60A |
| --- | --- |

| 2A - | Screw point radiator fan fuse for 2nd speed |
| --- | --- |

| 3 - | Terminal 30 screw point (battery) |
| --- | --- |

| 4 - | Terminal 30 screw point (starter) |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 31 |
| --- | --- | --- |

Position of fuses on fuse holder, in right luggage compartment

| A - | Heated rear window single fuse -S41- 40A |
| --- | --- |

| B - | Rear seat adjustment thermal fuse 1 -S335- 30A |
| --- | --- |

| q | up to model year 2005 |
| --- | --- |

| B - | Fuel pump fuse -S81- 40A |
| --- | --- |

| q | from model year 2006 |
| --- | --- |

| C - | Rear seat adjustment thermal fuse 2 -S336- 30A |
| --- | --- |

| D - | Vacant |
| --- | --- |

| D - | Communications fuse -S254-, 15A |
| --- | --- |

| q | up to model year 2005 |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 32 |
| --- | --- | --- |

Fuses on fuse holder, on right in luggage compartment

| 1 - | Terminal 30, fuse 1 -S131- 110A, for heated windscreen - Z2 |
| --- | --- |

| 1A - | Terminal 30a screw point heated windscreen -Z2- |
| --- | --- |

| 2 - | Terminal 30, fuse 2 -S132-, 60A for radio, navigation, TV tuner |
| --- | --- |

| 2A - | Terminal 30a screw point |
| --- | --- |

| 3 - | Terminal 30, fuse 3 -S133- 150A, for onboard supply |
| --- | --- |

| 3A - | Terminal 30a screw point (onboard supply) |
| --- | --- |

| 4 - | Terminal 30 screw point (fuse holder) |
| --- | --- |

| 5 - | Terminal 30 screw point (battery) |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 33 |
| --- | --- | --- |

Position of fuses (SF) on fuse holder, on right in luggage compartment, up to model year 2003

| - | SF1 - Convenience system central control unit -J393-, left light, 20A |
| --- | --- |

| SF2 - | Convenience system central control unit -J393-, right light, 10A |
| --- | --- |

| - | SF3 - Convenience system central control unit -J393-, power latching system, 20A |
| --- | --- |

| - | SF4 - Electric park and handbrake control unit -J540-, left parking brake motor -V282-, 30A |
| --- | --- |

| - | SF5 - Electric park and handbrake control unit -J540-, right parking brake motor -V283-, 30A |
| --- | --- |

| - | SF6 - Trailer detector control unit -J345-, left light, 15A |
| --- | --- |

| - | SF7 - Trailer detector control unit -J345-, right light, 15A |
| --- | --- |

| - | SF8 - Trailer detector control unit -J345-, 10A |
| --- | --- |

| - | SF9 - Fuel pump -G23-, 20A |
| --- | --- |

| - | SF10 - Fuel system pressurisation pump -G6-, 20A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 34 |
| --- | --- | --- |

Position of fuses (SF) on fuse holder, on right in luggage compartment, from model year 2004

| - | SF1 - Fuel pump -G23-, 20A |
| --- | --- |

| - | SF2 - Fuel system pressurisation pump -G6-, 20A |
| --- | --- |

| - | SF3 - Trailer socket - U10, terminal 15, 15A |
| --- | --- |

| q | vacant, from model year 2006 |
| --- | --- |

| - | SF4 - Interface control unit for vehicle location system -J843-, 5A |
| --- | --- |

| q | from June 2008 |
| --- | --- |

| - | SF5 - Convenience system central control unit -J393-, rear left light, 20A |
| --- | --- |

| - | SF6 - Convenience system central control unit -J393-, rear right light, 10A |
| --- | --- |

| - | SF7 - Convenience system central control unit -J393-, power latching system, 20A |
| --- | --- |

| - | SF8 - Control unit for electromechanical parking brake -J540-, left parking brake motor -V282-, 30A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 35 |
| --- | --- | --- |

| - | SF9 - Electric park and handbrake control unit -J540-, right parking brake motor -V283-, 30A |
| --- | --- |

| - | SF10 - Trailer detector control unit -J345-, left light, 15A/ 20A |
| --- | --- |

| q | 15A up to August 2007 |
| --- | --- |

| q | 20A from November 2007 |
| --- | --- |

| - | SF11 - Trailer detector control unit -J345-, right light, 15A/ 20A |
| --- | --- |

| q | 15A up to August 2007 |
| --- | --- |

| q | 20A from November 2007 |
| --- | --- |

| - | SF12 - Trailer detector control unit -J345-, 15A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 36 |
| --- | --- | --- |

Position of fuses (SF) on fuse holder, in right luggage compartment, right-hand drive

| 1 - | Fuel pump -G23-, 20 A |
| --- | --- |

| 2 - | Fuel system pressurisation pump -G6-, 20 A |
| --- | --- |

| 3 - | Vacant |
| --- | --- |

| 4 - | Vacant |
| --- | --- |

| 5 - | Convenience system central control unit -J393-, 20 A (rear left light) |
| --- | --- |

| 6 - | Convenience system central control unit -J393-, 10 A (rear right light) |
| --- | --- |

| 7 - | Convenience system central control unit -J393-, 20 A (power latching system) |
| --- | --- |

| 8 - | Control unit for electromechanical parking brake -J540-, left parking brake motor -V282-, 30 A |
| --- | --- |

| 9 - | Control unit for electromechanical parking brake -J540-, right parking brake motor -V283-, 30 A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 37 |
| --- | --- | --- |

| 10 - | Trailer detector control unit -J345-, 20A (left light) |
| --- | --- |

| 11 - | Trailer detector control unit -J345-, 20A (right light) |
| --- | --- |

| 12 - | Trailer detector control unit -J345-, 15 A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 38 |
| --- | --- | --- |

Position of fuses (SD) on fuse holder, on left in luggage compartment, up to model year 2003

| - | SD1 - Digital sound package control unit -J525-, 30A |
| --- | --- |

| - | SD2 - Navigation system with CD drive control unit -J401-, 5A |
| --- | --- |

| - | SD3 - TV-tuner -R78-, 10A |
| --- | --- |

| - | SD4 - Vacant |
| --- | --- |

| - | SD5 - Radio -R-, 5A |
| --- | --- |

| - | SD6 - Middle loudspeaker in rear shelf -R148-, 30A |
| --- | --- |

| - | SD7 - 12 V socket -U5-, 20A |
| --- | --- |

| - | SD8 - Remote control receiver for auxiliary heating -R64-, 5A |
| --- | --- |

| - | SD9 - Vacant |
| --- | --- |

| - | SD10 - Vacant |
| --- | --- |

| A1 - | Terminal 30 screw point (battery) |
| --- | --- |

| B1 - | Terminal 30 screw point |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 39 |
| --- | --- | --- |

Position of fuses (SD) on fuse holder, on left in luggage compartment, from model year 2004

| - | SD1 - Vacant |
| --- | --- |

| - | SD2 - Vacant |
| --- | --- |

| - | SD3 - Vacant |
| --- | --- |

| - | SD4 - Vacant |
| --- | --- |

| - | SD5 - Digital sound package control unit -J525-, 30A |
| --- | --- |

| - | SD6 - Navigation system with CD drive control unit -J401-, 5A |
| --- | --- |

| - | SD7 - TV-tuner -R78-, 10A |
| --- | --- |

| - | SD8 - Reversing camera system control unit -J772-, 5A |
| --- | --- |

| q | vacant, up to model year 2007 |
| --- | --- |

| - | SD9 - Radio -R-, digital radio -R147-, satellite radio -R146-, 5A |
| --- | --- |

| q | -R146- only USA |
| --- | --- |

| - | SD10 - Middle loudspeaker -R148-, 15A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 40 |
| --- | --- | --- |

| - | SD10 - Digital sound package control unit 2 -J787-, 30A |
| --- | --- |

| q | only BANG and OLUFSEN |
| --- | --- |

| - | SD11 - 12 V socket -U5-, 20A |
| --- | --- |

| - | SD12 - Remote control receiver for auxiliary heating -R64-, 5A |
| --- | --- |

| A1 - | Terminal 30 screw point (battery) |
| --- | --- |

| B1 - | Terminal 30 screw point (fuse holder) |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 41 |
| --- | --- | --- |

Position fuse holder on left in luggage compartment, right-hand drive

| 1 - | Vacant |
| --- | --- |

| 2 - | Vacant |
| --- | --- |

| 3 - | Vacant |
| --- | --- |

| 4 - | Vacant |
| --- | --- |

| 5 - | Digital sound package control unit -J525-, 30 A |
| --- | --- |

| 6 - | Navigation system with CD drive control unit -J401-, 5 A |
| --- | --- |

| 7 - | TV tuner -R78-, 10 A |
| --- | --- |

| 8 - | Reversing camera system control unit -J772-, 5 A |
| --- | --- |

| 9 - | Radio -R-, 5 A |
| --- | --- |

| - | Satellite radio -R146-, 5 A |
| --- | --- |

| - | Digital radio -R147-, 5 A |
| --- | --- |

| Audi A8 | Fitting Locations | No. 802 / 42 |
| --- | --- | --- |

| 10 - | Middle loudspeaker -R148-, 15 A |
| --- | --- |

| - | Digital sound package control unit 2 -J787-, 30 A |
| --- | --- |

| q | only BANG and OLUFSEN |
| --- | --- |

| 11 - | 12 V socket -U5-, 20 A |
| --- | --- |

| 12 - | Remote control receiver for auxiliary heating -R64-, 5 A |
| --- | --- |

| A1 - | Terminal 30 screw point (battery) |
| --- | --- |

| B1 - | Terminal 30 screw point (fuse holder) |
| --- | --- |