Servicing rear brakes Basic setting of rear wheel brake Handbrake cable must not be pretensioned when performing basic setting (release handbrake).

| To test: - ‒ → Use screwdriver to alternately press levers for handbrake cable against ... |  |
| --- | --- |

| - ‒ → ... stop (see arrow). If lever of opposite brake caliper is pulled off stop, then handbrake cable is excessively pretensioned. Note: For ease of illustration, stop is shown with brake caliper removed. |  |
| --- | --- |

| - ‒ → Loosen off the adjusting nut for the handbrake cable using special tool -3343- until both levers make contact with the stop of the brake caliper. |  |
| --- | --- |

Handbrake lever - Exploded view Removing and installing handbrake lever Removing - ‒ Remove rear exhaust on quattro vehicles. => 6-cylinder Engine, Mechanical Components; Repair group 26 => 8-cylinder Engine, Mechanical Components; Repair group 26 - ‒ Removing centre heat-insulation panels.

| - ‒ → Unscrew adjusting nut from connecting rod. - ‒ Detach compensator. - ‒ Detach bellows. - ‒ Removing centre console. => General Body Repairs; Repair group 70; Removing and installing centre console and handbrake lever trim |  |
| --- | --- |

| - ‒ Unscrew cheese head bolt -1- and take off clamp -2-. - ‒ → Disengage circlip -3- from pivot pin. - ‒ Press out pivot pin in direction of arrow. - ‒ Push handbrake lever slightly to rear and take out. Pay special attention to the following on installation: - ‒ Insert circlip carefully. - ‒ With the handbrake in the release position, secure the locking segment together with the clamp to the support bracket without lateral application of force. - ‒ Insert bellows carefully. - ‒ Ensure stress-free installation of exhaust system. - ‒ Adjusting handbrake => Page 46-76 |  |
| --- | --- |

Removing and installing handbrake cables Removing and installing handbrake cables Removing - ‒ Remove both rear wheels. - ‒ Release handbrake. - ‒ Remove rear exhaust on quattro vehicles. => 6-cylinder Engine, Mechanical Components; Repair group 26 => 8-cylinder Engine, Mechanical Components; Repair group 26 - ‒ Remove centre heat-insulation panels.

| Note: Mark the position at which the propshaft is bolted to the rear differential. - ‒ → Unscrew the bolts attaching the propshaft to the rear differential -1-. - ‒ Remove the heat insulation panel in the propshaft tunnel -arrow-. |  |
| --- | --- |

| - ‒ → Loosen the adjusting nut for the handbrake cable using special tool -3343-. - ‒ Take the handbrake cables off the compensating bar. - ‒ Pull out the retaining springs -arrow- from the handbrake cables. |  |
| --- | --- |

| - ‒ → Unhook the handbrake cable -3- from the brake caliper. - ‒ Press out the spring clip -2- and pull out the handbrake cable to the rear. Notes: - ◆ On FWD vehicles the open end of the spring clip -2- should point towards the brake. - ◆ Do not loosen bolt -1-. |  |
| --- | --- |

| - ‒ → Release retainer -1- and pull the handbrake cable out to the rear through the guide -2-. - ‒ Release attachments -4- and -5- far enough to allow the handbrake cable to be taken out. - ‒ Pull the handbrake cable out to the rear through the guide -3-. Pay special attention to the following on installation: - ‒ Propshaft to rear differential, 55 Nm. Remove any residue of microencapsulated adhesive from the internal threads of the rear differential. - ‒ Install and align the exhaust system. => 6-cylinder Engine, Mechanical Components; Repair group 26 => 8-cylinder Engine, Mechanical Components; Repair group 26 - ‒ Adjusting handbrake=> Page 46-76 |  |
| --- | --- |