Front seat covers and upholstery Removing and installing seat cover with sports seat Removing Important Observe safety precautions for working on airbag => - ‒ Remove front seat => Page 72-3.

| - ‒ → Detach fasteners -1- on left and right for seat extension. - ‒ Pull seat extension -3- out of retaining rods -2-. - ‒ Fold up seat extension with seat upholstery and lever out of clamping slot on back of seat frame. |  |
| --- | --- |

| - ‒ → Lever seat cover -2- out of fastening strip of seat extension. - ‒ Pinch off staples -1- and detach seat cover. |  |
| --- | --- |

| - ‒ → Lever seat cover -2- for side cushions out of clamping slot on side. - ‒ Pinch off staples -1- and pull off seat cover over side cushions. Installing Installation is carried out in the reverse order; note the following: - ‒ Use upholstery stapler V.A.G 1634 to attach staples to cover for side upholstery. |  |
| --- | --- |

| - ‒ → Use commercially available upholstery staple gun -1- to fasten cover. - ‒ Make use of upholstery staples as specified by staple gun manufacturer. Important Observe seat installation instructions for vehicles with side airbag =>Page 72-5. |  |
| --- | --- |