Removing and installing aluminium wheel bearing housing Servicing wheel bearing housing with bolted wheel bearing

| - Wheel bearing housing - ◆ Note the different running gear versions - ◆ Allocation => Parts Catalogue - Hemispherical collared bolts: 80 Nm + further 90° turn. - Bushes - Cover plate - Hexagon bolt, 10 Nm |  |
| --- | --- |

| - Wheel hub - ◆ Pressing out =>Fig.1 - ◆ Pressing off bearing inner race =>Fig.2 - ◆ Stepped version - ◆ The bearing/wheel hub contact surfaces and threaded holes must be free of paint and dirt - ◆ Lightweight wheel hub => Page 40-58 - Ball bearing - ◆ Removing and installing with wheel bearing housing installed => Page 40-50 - ◆ Installation position: - ‒ Large internal diameter of wheel bearing faces toward wheel hub - ◆ Pressing off bearing inner race =>Fig.2 Removing - ‒ Remove wheel trim; on light-alloy wheels, detach cover cap (puller in vehicle tool kit). - ‒ Unscrew hexagon bolt for drive shaft. (Loosen only when vehicle is standing on wheels -danger of accident-.) Note =>Page 40-105 - ‒ Remove wheel. - ‒ Secure brake disc with a wheel bolt. |  |
| --- | --- |

| - ‒ Unscrew hexagon bolt -2-. - ‒ Pull ABS speed sensor wiring out of retainer on wheel bearing housing. - ‒ → Remove bolts -1- for brake caliper and remove brake caliper. - ‒ Tighten hexagon bolt -2-. Note: The brake caliper must not be reattached to e.g. the body before the brake line holder has not yet been bolted on using bolt -2-. |  |
| --- | --- |

| - ‒ → Unscrew bolt -1- for ABS speed sensor. |  |
| --- | --- |

| Notes: - ◆ Fasten brake caliper to body using wire. - ◆ Do not suspend the brake caliper from the brake hose. - ‒ Remove brake disc. |  |
| --- | --- |

| - ‒ → Unscrew nut -1-, remove hexagon bolt and pull out both links -2- in upwards direction. The slots in the wheel bearing housing must not be widened using a chisel or similar. - ‒ Swivel wheel bearing housing aside in direction of arrow while pulling drive shaft pin out of wheel hub in the process. - ‒ Remove wheel bearing housing. |  |
| --- | --- |

| - ‒ Unscrew hemispherical collared bolts -1-. - ‒ Remove wheel bearing unit. |  |
| --- | --- |

| → Fig.1 Pressing out wheel hub from wheel bearing |  |
| --- | --- |

| → Fig.2 Pressing bearing inner race off wheel hub - ‒ Attach separating tool into circular groove of bearing inner race -arrow- and pre-tension with spindle. Note: Use commercially available separating device e.g. Kukko 15-17. |  |
| --- | --- |

| - ‒ → Press bearing inner race from wheel hub. |  |
| --- | --- |

| → Fig.3 Pressing wheel hub into wheel bearing - ‒ When pressing in, thrust pad -VW 511- must only make contact with inner race. Installing |  |
| --- | --- |

| - ‒ → Hemispherical collared bolt tightening torque -1-: 80 Nm and turn +90° further - ‒ Push outer joint of drive shaft into wheel hub and hand-tighten new hexagon bolt. - ◆ Remove any adhesive residue from thread of joint pin. - ‒ Insert both upper transverse link joint pins into wheel bearing housing. |  |
| --- | --- |

| - ‒ → Tighten new self-locking nuts -1- to 40 Nm. - ‒ When tightening, press down upper transverse link as far as possible. - ‒ Install ABS speed sensor. - ‒ Fit brake disc, bolt on brake caliper and tighten to 190 Nm. - ‒ Fit wheel and tighten to 120 Nm. - ‒ Tighten drive shaft hexagon bolt. Only tighten when vehicle is standing on wheels -danger of accident-. - Tightening torques: - Hexagon bolt, 190 Nm + 180° further turn Note =>Page 40-107 The alignment of the front axle must be checked and if necessary adjusted using VW/Audi-approved wheel alignment equipment. |  |
| --- | --- |